Team Hot Glue

Engineering Journal

WRO 2026

Lukas Mažrimas, Jokūbas Buta, Jokūbas Dargevičius

Table of Contents

Preface	2

1. Structuring, workflow, concept	3

1.1. Team structure and workflow	3

1.2. Design process	3

1.3. Design philosophy	4

2. Mechanical platform	5

2.1. Overview	5

2.2. Steering	5

3. Circuit design and sensors	6

4. Software	7

# Preface

This document is designed to explain some key aspects of the design of our team’s robot. Before beginning, there is, in our opinion, benefit in explaining some terminology and overall document’s structure first:

During the design process, the robot acquired an informal name, “Bobotas”. This came from an inside joke in a different (Lithuanian) language and explaining it would be a long and complicated endeavor. What we want to highlight, though, is that we will use this name translated to English (“Bobot”) alongside a simple “robot” when referring to our robot.

The journal has a couple of distinct chapters, each explaining a different area of the robot’s development. Moreover, the chapters also showcase the chronological order of the development, meaning, that, for example, the mechanical platform was first created and then all the sensors and circuits were tailored to the existing platform. The reasoning behind this will be explained in chapter 1.

# 1. Structuring, workflow, concept

## 1.1. Team structure and workflow

Before starting any meaningful work, we took some time to map out the general ideas and assign each of us a part of the design process. Lukas was responsible for the mechanical platform, Jokūbas Buta for the general movement programming and integration, and Jokūbas Dargevičius for circuits and sensors as well as their programming.

There was, of course, a great deal of team effort and solving problems together, but giving primarily one type of work to each team member was beneficial and efficient for a multitude of reasons. First, we all come from different backgrounds and have different types of experience and skillsets. Therefore, it makes sense to have the person, who is best at coding, design most of the software. It also makes sense having all the soldering done by the person who has the most experience with soldering. Moreover, a mechanical platform will be better if made by one most proficient in physics. Second, creating a robot is no simple feat and each of us learned a lot in the process. We found it much easier and better for us to each concentrate on one particular field and do lots of in-depth research in that particular field, rather than us all trying to juggle everything at once.

## 1.2. Design process

Another thing we did in the beginning was decide on the specific electrical components and sensors to use and buy them. More about the exact choices will be explained in chapter 3, but knowing all the components in advance allowed for a better overall design and allowed us time to prepare and find good hardware and software solutions for the particular “stock” components.

We chose to follow a simple and linear development path: chose and buy the sensors, then make a mechanical platform, then make all the circuits and mount them and the sensors on the chassis, and, finally, program everything. This path was easy to structure and follow. However, it also created a double challenge: lots of foresight must be put into the design in the beginning, and later design elements must be flexible enough to work around existing limitations. We tried as best we can to mitigate these issues by brainstorming and agreeing on the general concept and design decisions in advance.

## 1.3. Design philosophy

One of the main driving principles behind most of the design decision-making was that of the F1 design legend Collin Chapman: “Simplify, then add lightness”. We wanted to have as little moving parts, as possible, because moving parts wear and can ultimately break. We also followed another principle of his: “Rules are for the obedience of fools and interpretations of smart men.” This is to say, we were on the outlook for areas of excessive design freedom.

# 2. Mechanical platform

## 2.1. Overview

The mechanical platform consists of three main areas: steering system, driving axle and the chassis that connects them. We agreed to make a conventional rear-wheel-drive system: front wheels are designated to steering, and rear wheels are driven. This type of package can be seen all throughout the automotive industry, as it is easy to package and simple and effective.

## 2.2. Steering

# 3. Circuit design and sensors

# 4. Software